Palm Oil Specification

- Palm Acid Oil (PAO)

|  | FFA (As Palmitic) | 50% min. |
| --- | --- | --- |
|  | M&I | 3.0% max. |
|  | Total Fatty Matter | 95% min. (basic 97%) |

- Palm Fatty Acid Distillate (PFAD)

|  | FFA (As Palmitic) | 70% min. |
| --- | --- | --- |
|  | M&I | 1.0% max. |
|  | Saponifiable Matter | 95% min. (basic 97%) |

- RBD Palm Kernel Oil (PKO)

|  | FFA (as Lauric) | 0.1% max. |
| --- | --- | --- |
|  | M&I | 0.1% max. |
|  | Iodine Value (VJJS) | 19 max. |
|  | Color (5.25” lovibond cell) | 1.5red/12yellow max. |

- RBD Palm Stearin (PKO)

|  | FFA (as Palmitic) | 0.2% max. |
| --- | --- | --- |
|  | M&I | 0.15% max. |
|  | Iodine Value (VJJS) | 48 max. |
|  | Melting Point | 44 max. |
|  | Color (5.25” lovibond cell) | 3.0red/20yellow max. |

| Palm Oil Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | STANDARD PORAM & MEOMA SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CODE | PRODUCT DESCRIPTION |  | FFA | MNI |  |  | IV |  | MP |  |  | DOBI |  | COLOUR R/Y | CLOUD PT | SAP |  | TFM | PPM |
| OTSB1040 | CRUDE PALM OIL (CPO) |  | 5% | 0.0025 |  |  |  |  |  |  |  | 2.3-2.4 |  |  |  |  |  |  |  |
| OTSB1041 | RBD PALM OIL AS PLAMITIC |  | 0.10% | 0.10% |  |  | 50-55 |  | 33-39 |  |  |  |  | 3R |  |  |  |  |  |
| OTSB1042 | RBD PALM OLEIN |  | 0.10% | 0.10% |  |  | 56 MIN |  | 24 MAX |  |  |  |  | 3R |  |  |  |  |  |
| OTSB1043 | RBD PALM STEARINE |  | 0.20% | 0.15% |  |  | 48 MAX |  | 44 MIN |  |  |  |  | 3R |  |  |  |  |  |
| OTSB1044 | PALM FATTY ACID DISTILLATE (PFAD) |  | 70 % MIN | 1.0% |  |  |  |  |  |  |  |  |  |  |  | 95 % MIN |  |  |  |
| OTSB1045 | PALM ACID OIL (PAO) |  | 70 % MIN | 3.0% |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 % MIN |  |
| OTSB1046 | CRUDE PALM KERNEL OIL AS LAURIC (CPKO) |  | 0.10% | 0.10% |  |  | 19 MAX |  |  |  |  |  |  | 1R |  |  |  |  |  |
| OTSB1047 | RBD PALM KERNEL OIL |  | 0.10% | 0.10% |  |  | 17-18 |  | 24 MIN |  |  |  |  | 1R/10Y |  |  |  |  |  |
| OTSB1048 | RBD PALM KERNEL OLEIN |  | 0.10% | 0.10% |  |  | 22 MIN |  | 24 MIN |  |  |  |  | 1R/10Y |  |  |  |  |  |
| OTSB1049 | RBD PALM KERNEL STEARINE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OTSB1050 | RBD PALM KERNEL FATTY ACID DISTILLATE (PKFAD) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OTSB1051 | MARGARINE |  | 0.10% | 16.00% |  |  |  |  | 33-40 |  |  |  |  | YELLOW |  |  |  |  | 200 MAX |
| OTSB105 | VEGETABLE GHEE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OTSB1053 | VEGETABLE SHORTENINGS |  | 0.10% | 0.10% |  |  | 40-50 |  | 40 |  |  |  |  | 2.5R/25Y |  |  |  |  |  |
| OTSB1054 | DOUGH FATS |  | 0.10% | 0.10% |  |  | 33-39 |  | 46-51 |  |  |  |  | 3R |  |  |  |  | 200 MAX |
| PORAM |  | PALM OIL REFINERS ASSOCIATION MALAYSIA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MEOMA |  | MALAYSIAN EDIBLE OILS MANUFACTURERS ASSOCIATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FFA |  | FREE FATTY ACID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MNI |  | MOISTURE & IMPURITIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IV |  | IODINE VALUE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MP |  | MELTING POINT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DOBI |  | DETERIORATION OF BLEACHIBILITY INDEX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R/Y |  | RED/YELLOW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SAP |  | SAPONIFIABLE MATTER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TFM |  | TOTAL FATTY MATTER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PPM |  | PERMITTED ANTIOXIDANTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |